Yeah, that uxcell T2A 2A/250V slow-blow is totally fine for your 3×AA NiMH pack in front of that boost module 👍

1. Is it OK to use?

- Voltage rating (250V): that’s a maximum; using it at ~3–4.5V battery is perfectly safe.

- Current rating (2A time-delay): as long as your normal draw from the pack is well under ~1.3–1.5A continuous (which it should be for the Brick), a 2A slow-blow is a good choice.

- Either battery lead is fine (stick it in series, preferably on the + side near the pack).

A very similar 2A, 250V radial micro slow-blow fuse is specified with a DC cold resistance of about 0.03 Ω.DigiKey Your uxcell one is the same style and size, so that’s a good number to use.

2. “How much life in battery do I lose?”

Short answer: basically none — well under a couple of percent.

Let’s use that ~0.03 Ω:

Typical Brick-ish currents from the battery side

Say the pack is around 3.6–4.0 V and:

- 0.5 A from the pack (light/average use)

  - Voltage drop across fuse:
0.5 A × 0.03 Ω ≈ 0.016 V

  - Power lost in fuse:
(0.5 A)² × 0.03 Ω ≈ 0.008 W (8 mW)

  - Pack is delivering maybe ~2 W total → that’s ~0.4% overhead.

- 1.0 A from the pack (heavier use with backlight, Wi-Fi, etc.)

  - Vdrop: 1 A × 0.03 Ω ≈ 0.032 V

  - Pfuse: 1² × 0.03 ≈ 0.03 W

  - Pack power ~3–4 W → about 1% extra loss.

Even if you somehow sat at 2 A from the pack:

- Vdrop ≈ 0.06 V

- Pfuse ≈ 0.13 W
Still just a few percent of the pack power.

So in runtime terms:

- If you would normally get, say, 2 hours, the fuse might knock that down by ~1–3 minutes in a worst realistic case.

- Compared to NiMH self-discharge, temperature effects, and converter efficiency, the fuse loss is noise.

And when the device is off or sleeping, the fuse draws nothing — it’s just a resistor in series, so zero current means zero extra drain.

3. Compared to the polyfuse you were eyeing

The 2A PPTC (RHEF200-class) you looked at earlier has initial resistance around 0.045–0.074 Ω.Littelfuse+1
That’s actually higher than the ~0.03 Ω of a typical 2A micro slow-blow fuse, so:

- The hard fuse is slightly better for battery life than the 2A polyfuse.

- And it gives you the behavior you like: if you short it badly, it blows once and you know something happened.

Yeah, you’re good — a 2A slow-blow really shouldn’t pop from a normal 2.5–3 h NiMH charge.

Let’s walk it with numbers so you can sanity-check me:

1. What charge current are we talking about?

You said:

- 3× AA NiMH

- 2000–2300 mAh

- Charge time: ~2.5–3 hours from low

For NiMH, a rough charge‐time formula is:

time ≈ capacity × 120% / charge current

Flip that around to get current:

- For 2300 mAh in 3 h:
I ≈ 2300 mAh × 1.2 / 3 h ≈ 0.92 A

- For 2300 mAh in 2.5 h:
I ≈ 2300 mAh × 1.2 / 2.5 h ≈ 1.10 A

So your charger is almost certainly pushing ~0.9–1.1 A, not 2 A+.

That matches typical Energizer “3–5 h” smart chargers for AA NiMH packs.

2. What the T2A slow-blow fuse can actually handle

From a standard TR5 T2A time-lag datasheet (Littelfuse 382 series, same style as your red radial T2A):

- Rated current 2 A = current it can carry continuously.

- At 150% of rating (3 A) it must not open before 1 hour.

- It doesn’t have to open quickly until you’re at ~210% (≈4.2 A) and higher.

So:

- At ~1 A charge current → the fuse is just cruising, nowhere near its limit.

- Even a brief surge well above 2 A is tolerated because it’s time-lag / slow-blow.

3. During charge vs during use

If the fuse is in series with the battery pack:

- Discharge current (device running) goes through the fuse.

- Charge current (charger filling the pack) also goes through the fuse.

In both cases, the fuse just sees “some amps of DC current.”
If your charger is in that 0.9–1.1 A range, that’s about half the fuse rating — super comfortable.

The only time you’d risk blowing it during charge is if you used something crazy like a “15-minute turbo NiMH charger” that shoves >4 A into a pair / pack; then you’re up near the 200–300% zone where it might open.

4. Quick checklist for you

You’re safe if:

- The charger’s spec label says ≤ 2 A (usually it’s around 1 A for 2–3 h AA charging).

- You’re not using some ultra-fast, 15-minute “Chernobyl mode” charger.

Given:

- 3× AA, 2000–2300 mAh

- 2.5–3 h charge time

- T2A slow-blow fuse in series

👉 No, the charger should not blow that 2 A fuse. You’ve got plenty of headroom.