# Making a GIGA-like STM32H747 Board Arduino-Compatible Without Bootloader or Mbed

## 1. Arduino Compatibility Does NOT Require:

• Arduino bootloader

• Arduino GIGA core

• MbedOS

• Any proprietary Arduino code

These are optional layers Arduino added, not what defines compatibility.

## 2. What Actually Makes a Board Arduino-Compatible:

1. A valid Arduino core (STM32duino – open-source).

2. A board definition (variant.cpp, pins_arduino.h, memory layout).

3. A working upload method (ST-Link or DFU).

None of these require an Arduino bootloader.

## 3. The Arduino Bootloader Is NOT Required:

Bootloader only provides USB serial upload, reset over USB, auto-DFU entry, and proprietary partitioning.

Removing it does NOT prevent Arduino compatibility.

Upload via ST-Link or DFU instead.

This is identical to how STM32 Nucleo boards work.

## 4. STM32H747 Support in STM32duino:

• Dual-core H747 support already implemented.

• Full flash/GPIO/RCC/HAL integration.

• USB CDC works with firmware initialization.

• All key peripherals (SPI/I2C/UART/DMA) supported.

• SDRAM + QSPI can easily be added.

Brick's M7-only execution is fully supported.

## 5. Creating Your Own Variant:

In STM32duino, create folder:

variants/BRICK_H747/

Place inside:

• variant.cpp

• pins_arduino.h

• PeripheralPins.c

Define:

• Pin mappings

• Flash/RAM sizes

• Clock configuration

• Boot address (0x08000000)

Then PlatformIO can compile Arduino code for your clone like it does for the GIGA.

## 6. Uploading Without Bootloader:

Valid upload methods without Arduino bootloader:

• ST-Link (best)

• DFU (ROM bootloader)

• J-Link

• OpenOCD

This is normal and widely used.

## 7. USB Serial Considerations:

USB CDC will not appear automatically without Arduino bootloader.

BUT:

• USB CDC can be enabled entirely in firmware.

• STM32duino already supports CDC.

• You will still get /dev/ttyACM0 or COMx.

No Arduino bootloader needed for USB logs.

## 8. Mbed Is Irrelevant:

GIGA core uses Mbed internally for convenience.

Your firmware does not depend on it.

Your STM32H747 clone does NOT need Mbed for Arduino compatibility.

## 9. Practicality Summary:

YES — Practical

YES — Standard industry practice

YES — Legal

YES — Supported by PlatformIO

YES — Brick firmware ports with minor adjustments

NO — Proprietary GIGA drivers will not be available

NO — Arduino branding cannot be used without licensing

But:

✔ 100% Arduino-compatible clone is achievable

✔ No bootloader required

✔ No licensing cost

✔ No hidden obstacles

## Final Answer (Direct):

YES — You can make your STM32H747 clone fully Arduino-compatible with:

• No Arduino bootloader

• No Mbed

• No proprietary Arduino core

Just use:

→ STM32duino core

→ Custom variant definition

→ ST-Link or DFU upload method

This is exactly how non-Arduino STM32 boards support Arduino today.