# Scaling Up: STM32 MCU and WiFi/Cellular Modules for Bulk Production

# 1. MCU Selection for Scaling Up

• For volume production beyond Arduino GIGA, the recommended MCU family is STM32H7, particularly with M7 core (e.g., STM32H747, STM32H750, STM32H7A3). These match GIGA-level performance and are supported by all major distributors.
• Order in trays or reels from distributors: Mouser, Digi-Key, Arrow, Avnet, Future Electronics.

# 2. Recommended WiFi/Cellular Modules (Bulk, Commercial)

A. WiFi Modules
• u-blox NORA-W106 (used in GIGA): Modular certified, available in bulk, global supply.
• Espressif ESP32/ESP32-S3 modules: Cheap, modular certified, industrial supply.
• Murata Type 1DX/1LV: Industrial WiFi/BT, certified, widely used in commercial products.

B. Cellular Modules
• u-blox SARA series: LTE-M/NB-IoT, Cat M1, modular certified, carrier-ready.
• Quectel BG96/EC25/EG91: All major carriers, FCC/CE certified, used in commercial IoT.
• Other: Telit, Sierra Wireless, SIMCom – for North America/Europe, all modular certified.

# 3. Bulk/Industrial Suppliers

• Mouser (US/global)
• Digi-Key (US/global)
• Arrow Electronics
• Avnet
• Future Electronics
• For high volume: direct from manufacturer (u-blox, Espressif, Quectel, etc.)

# 4. FCC/CE Compliance with Modular Certified Radios

• If you use a pre-certified WiFi or cellular module (e.g., u-blox NORA-W106, ESP32, Quectel BG96) and follow the manufacturer's integration and antenna guidelines, you are covered by their FCC/CE modular certification for the radio portion.
• You must:
   – Clearly label your product with “Contains FCC ID: ...”
   – Follow antenna placement and reference design exactly
• You are responsible for unintentional emissions (Part 15B) from your custom board (basic digital emissions scan recommended for 500+ units).

# 5. Bulk Integration and Supply Tips

• Stick to pre-certified modules for WiFi/Cellular to simplify compliance and speed time to market.
• Use industrial distributors for stable supply, volume pricing, and documentation.
• Keep full records of FCC/CE/IC IDs and integration docs for every part.
• If you want cellular carrier approval (for 1000s of units), check PTCRB and carrier requirements.

# 6. Summary Table

| Component | Example Part | Bulk Supplier | FCC/CE Status | Notes |
| --- | --- | --- | --- | --- |
| MCU | STM32H747XIH6 | Mouser, Digi-Key | N/A (MCU only) | Order in trays or reels |
| WiFi | u-blox NORA-W106 | Mouser, Digi-Key | Modular Cert | Label with FCC ID |
| WiFi | ESP32-S3 module | Mouser, Digi-Key | Modular Cert | Low cost, widely used |
| Cellular | Quectel BG96 | Mouser, Avnet | Modular Cert | Carrier/PTCRB for mass deployment |
| Cellular | u-blox SARA-R4 | Mouser, Digi-Key | Modular Cert | Global variants available |

For 500+ units or new geographies, always confirm current regulatory requirements and consider a quick emissions scan for your assembled product.